Documentation

IPv6

with ACLS, DHCPV6 UND OSPFV3

| Created by Created on the Learning Field 9 | Niklas Marsollek, Louis Scheiwe, Marcel Stein (IT4o) 23.04.2025, Hamburg Integrate Clients into a Network (IP6v) |
| --- | --- |

Contents

1. Introduction - IPv6 Implementation and Network Services 
1.1. Requirements 
1.2. Initial Configuration Steps 
2. IPv6 Addressing Scheme 
2.1. IPAM - Sites and Local Networks 
2.2. IPAM - WAN Transport Networks 
2.3. IPAM - Switch Management Addresses 
3. IPv6 Access Control Lists (ACLs) 
3.1. ACL Requirements
3.2. Example ACL Configuration
3.3. Additional Security Rule
4. DHCPv6
4.1. SLAAC with DHCPv6 Server
4.2. DHCPv6 Stateful
4.3. Central DHCPv6 Server
5. OSPFv3
5.1. Dynamic Routing with OSPFv3
5.2. Example Configuration
5.3. Verification

# introduction – ipv6 implementation and network services

This documentation describes the implementation of IPv6 network services in a multi-site environment. The focus is on IPv6 addressing, Access Control Lists (ACLs), DHCPv6, OSPFv3, and structured IP address management (IPAM).

The goal is to provide a clear and practical documentation for setting up, securing, and maintaining an IPv6-based infrastructure across multiple locations.

## REQUIREMENTS 1

- A functioning network topology with routers, switches, clients, and servers

- A documented network structure and addressing plan

- IPv6-capable devices and software

- Administrative access to all relevant routers and switches

- Defined communication requirements between sites and services

- Planning and preparation completed in advance

## 1.2. INitial configuration steps 1

- Plan the IPv6 address structure for all sites, VLANs, and WAN links

- Configure IPv6 routing on all routers

- Assign IPv6 addresses to all LAN and WAN interfaces

- Configure gateway addresses for each VLAN

- Define switch management addresses

- Create IPv6 ACLs according to the security requirements

- Configure DHCPv6 for automatic client configuration

- Enable OSPFv3 for dynamic routing between all locations

- Verify connectivity, routing, and service accessibility

# ipv6 adressing scheme 2

The following section presents the IPv6 addressing concept used for the network. It documents the logical allocation of IPv6 addresses to client networks, server networks, WAN transport links, and management interfaces.

## IPAM – SITES AND LOCAL NETWORKS 2

| Site / LAN | Purpose | IPv6 Network Address | Gateway (Router) | Example End Device |
| --- | --- | --- | --- | --- |
| Hamburg (VLAN 10) | Client Network 1 | 2001:db8:10::/64 | 2001:db8:10::1 | 2001:db8:10::10 (PC2) |
| Hamburg (VLAN 20) | Client Network 2 | 2001:db8:20::/64 | 2001:db8:20::1 | 2001:db8:20::10 (PC3) |
| Lübeck (VLAN 30) | Client Network 1 | 2001:db8:30::/64 | 2001:db8:30::1 | 2001:db8:30::10 (PC0) |
| Lübeck (VLAN 40) | Client Network 2 | 2001:db8:40::/64 | 2001:db8:40::1 | 2001:db8:40::10 (PC1) |
| Berlin (VLAN 50) | Server Network | 2001:db8:50::/64 | 2001:db8:50::1 | 2001:db8:50::100 (Server0) |
| Munich (VLAN 60) | Server Network | 2001:db8:60::/64 | 2001:db8:60::1 | 2001:db8:60::100 (Server1) |

| WAN Connection | IPv6 Network Address | IP Address Side A | IP Address Side B |
| --- | --- | --- | --- |
| Hamburg -> Lübeck | fd00:0:0:1::/64 | fd00:0:0:1::1 (Router1) | fd00:0:0:1::2 (Router2) |
| Hamburg -> Berlin | fd00:0:0:2::/64 | fd00:0:0:2::1 (Router1) | fd00:0:0:2::2 (Router3) |
| Hamburg -> Munich | fd00:0:0:3::/64 | fd00:0:0:3::1 (Router1) | fd00:0:0:3::2 (Router4) |
| Lübeck -> Berlin | fd00:0:0:4::/64 | fd00:0:0:4::1 (Router2) | fd00:0:0:4::2 (Router3) |
| Lübeck -> Munich | fd00:0:0:5::/64 | fd00:0:0:5::1 (Router2) | fd00:0:0:5::2 (Router4) |
| Berlin -> Munich | fd00:0:0:6::/64 | fd00:0:0:6::1 (Router3) | fd00:0:0:6::2 (Router4) |

## IPAM – wan transport networks 2

## IPAM – switch management ADRESSES

| Switch ID | Site | Management IP (VLAN Interface) | Default Gateway |
| --- | --- | --- | --- |
| Multilayer Switch3 | Hamburg | 2001:db8:10::fe | 2001:db8:10::1 |
| Multilayer Switch4 | Lübeck | 2001:db8:30::fe | 2001:db8:30::1 |
| Multilayer Switch2 | Berlin | 2001:db8:50::fe | 2001:db8:50::1 |
| Multilayer Switch1 | Munich | 2001:db8:60::fe | 2001:db8:60::1 |

# ipv6 access control list (acls)

To improve network security, IPv6 ACLs are used. These allow specific control of traffic between sites and services.

## acl requirements

Must-have

- SSH access to network devices must only be allowed from clients at the Hamburg site

- Clients at the Hamburg site must be allowed to access the web servers via HTTP (Port 80) and HTTPS (Port 443)

- Clients at the Lübeck site must only be allowed to access the web servers via HTTPS (Port 443)

- All configurations must be documented

## EXAMPLE ACL CONFIGURATIOn

SSH access only from Hamburg

plaintext

ipv6 access-list SSH-HAMBURG permit tcp 2001:db8:10::/64 any eq 22 deny tcp any any eq 22 permit ipv6 any any

The ACL is then applied on the target router interface:

plaintext

interface GigabitEthernet0/0 ipv6 traffic-filter SSH-HAMBURG in

HTTP and HTTPS from Hamburg to the web server

plaintext

ipv6 access-list WEB-HAMBURG permit tcp 2001:db8:10::/64 host 2001:db8:100::10 eq 80 permit tcp 2001:db8:10::/64 host 2001:db8:100::10 eq 443

HTTPS only from Lübeck to the web server

plaintext

ipv6 access-list WEB-LUEBECK permit tcp 2001:db8:20::/64 host 2001:db8:100::10 eq 443 deny tcp 2001:db8:20::/64 host 2001:db8:100::10 eq 80 permit ipv6 any any

## ADDITIONAL security

Additional security measures should be implemented to protect the IPv6 infrastructure against unauthorized access and common network attacks.

### Recommended Security Rules

#### Block Telnet Access

should not be used because it transmits credentials in clear text.

ipv6 access-list BLOCK-TELNET
 deny tcp any any eq 23
 permit ipv6 any any

#### Restrict ICMPv6 Traffic

ICMPv6 is required for Neighbor Discovery and Path MTU Discovery. Therefore, only unnecessary ICMPv6 types should be filtered.

ipv6 access-list ICMPV6-PROTECTION
 permit icmp any any nd-na
 permit icmp any any nd-ns
 permit icmp any any router-advertisement
 permit icmp any any router-solicitation
 permit icmp any any echo-request
 permit icmp any any echo-reply
 deny ipv6 any any log

#### Secure Management Access

line vty 0 4
 transport input ssh
 login local

## DHCPV6

DHCPv6 provides automatic IPv6 address assignment and additional network parameters to end devices.

4.1. SLAAC WITH DHCPv6 SERVER

In this deployment model, clients generate their own IPv6 addresses using Stateless Address Autoconfiguration (SLAAC). The DHCPv6 server only provides additional information such as DNS servers.

Router Configuration

interface GigabitEthernet0/0
 ipv6 address 2001:db8:10::1/64
 ipv6 nd other-config-flag
 ipv6 dhcp server DHCP-SLAAC

DHCPv6 Pool

ipv6 dhcp pool DHCP-SLAAC
 dns-server 2001:db8:50::100
 domain-name company.local

Advantages

- Minimal administration effort

- Automatic address generation

- Suitable for client networks

4.2. DHCPv6 STATEFUL

In Stateful DHCPv6 mode, the DHCP server assigns and tracks all IPv6 addresses.

Router Configuration

interface GigabitEthernet0/0
 ipv6 address 2001:db8:20::1/64
 ipv6 nd managed-config-flag
 ipv6 dhcp server DHCP-STATEFUL

DHCPv6 Pool

ipv6 dhcp pool DHCP-STATEFUL
 address prefix 2001:db8:20::/64
 dns-server 2001:db8:50::100
 domain-name company.local

Advantages

- Central address management

- Address tracking possible

- Suitable for servers and managed devices

## DHCPV6

In Stateful DHCPv6 mode, the DHCP server assigns and tracks all IPv6 addresses.

Router Configuration

interface GigabitEthernet0/0
 ipv6 address 2001:db8:20::1/64
 ipv6 nd managed-config-flag
 ipv6 dhcp server DHCP-STATEFUL

DHCPv6 Pool

ipv6 dhcp pool DHCP-STATEFUL
 address prefix 2001:db8:20::/64
 dns-server 2001:db8:50::100
 domain-name company.local

Advantages

- Central address management

- Address tracking possible

- Suitable for servers and managed devices

## CENTRAL DHCPv6 SERVER

The DHCPv6 service is hosted centrally in the Berlin server network.

| Service | IPv6 Address |
| --- | --- |
| DHCPv6 Server | 2001:db8:50::100 |
| DNS Server | 2001:db8:50::101 |
| ## DHCPv6 relay configuration |  |

interface GigabitEthernet0/0
 ipv6 dhcp relay destination 2001:db8:50::100

Verification

show ipv6 dhcp interface
show ipv6 dhcp binding
show ipv6 dhcp pool

## verification

VERIFICATION

Neighbor Relationships

show ipv6 ospf neighbor

Expected result:

Neighbor ID     State
2.2.2.2         FULL
3.3.3.3         FULL
4.4.4.4         FULL

Routing Table

show ipv6 route ospf

OSPF Interface Status

show ospfv3 interface

Connectivity Test

ping ipv6 2001:db8:50::100
traceroute ipv6 2001:db8:60::100

Successful communication between all locations confirms proper OSPFv3 operation.

## network testing

Verify communication between:

- Hamburg ↔ Lübeck

- Hamburg ↔ Berlin

- Hamburg ↔ Munich

- Lübeck ↔ Berlin

- Lübeck ↔ Munich

- Berlin ↔ Munich

Example:

ping ipv6 2001:db8:50::100

## CONCLUSION

The IPv6 implementation provides a scalable, secure, and centrally managed network infrastructure across all locations. Through structured IPv6 addressing, DHCPv6 automation, ACL-based access control, and OSPFv3 dynamic routing, the network supports reliable communication while maintaining security and operational efficiency. The documented IPAM structure simplifies future expansion and ongoing administration.